DECLARATION OF COMPLIANCE

| Description | Material | Article Number |
| --- | --- | --- |
| Sealing film | PP | 107074 |

Duni declares that the article meets the requirements of:

- EU Regulation 1935/2004/EC (Framework regulation)

- EU Regulation 2023/2006/EC (GMP)

- EU Regulation 10/2011/EC with amendments (Plastic regulation)

Overall migration (1)

According to the above mentioned regulations, the overall migration does not exceed 10 mg/dm² or 60 mg/kg.

Specific migration (2)

This article contains monomers or additives subject to restrictions under the plastic regulation 10/2011 and its amendments. A specific migration test proves that these are within the limits. If more information regarding the monomers or additives are needed please contact Duni AB.

Area of use

Based on the migration tests and Declaration of Compliance, the articles can be used safely with all types of food. This film is suitable for use under the following conditions:

- Long term storage above 6 months at room temperature or below

- Heating up to:

  - 121°C for up to 2 hours

- Microwave oven

This film is not suitable for use in conventional oven.

Test conditions

Migration tests on the article material performed by an independent institute showed that under the following test conditions, overall migration (see 1.) falls considerably below the limit given by regulation 10/2011.

Overall migration

10 % Ethanol	2 hours at 121°C +10 days at 60°C

3% Acetic Acid	2 hours at 121°C +10 days at 60°C

Vegetable oil	2 hours at 121°C +10 days at 60°C

Specific migration

10 % Ethanol	2 hours at 121°C +10 days at 60°C

3% Acetic Acid	2 hours at 121°C +10 days at 60°C

Vegetable oil	2 hours at 121°C +10 days at 60°C

The ratio of food contact surface area to volume used is 6 dm²/kg

The following additives are also authorized as food additives by Regulation (EC) No 1333/2008 or as flavorings by Regulation (EC) No 1334/2008 and according to our raw material suppliers may be present in the substances used in the film.

| Substance | E number / PM ref. number |
| --- | --- |
| Polyoxyethylene sorbitan monolaurate | E 432 / PM 79040 |
| Silicon dioxide | E 551 / PM 86240 |
| Talc | E 553b / PM 92080 |
| Dimethylpolysiloxane | E 900 / PM 76721 |

Please be advised that Duni AB does not add anything into the product.

This document of compliance is based on:

- Documentation from suppliers

- Overall migration test

- Specific migration

This document was issued electronically and is therefore valid without signature.

It is responsibility of the packer of the finished packages to ensure that the package is safe to use in the intended conditions (W/min) taking into account all relevant information e.g. the shape and size of the package and packaged food.