DECLARATION OF COMPLIANCE

| Description | Material | Article Number |
| --- | --- | --- |
| Cling Film | PVC | 716000 |

Duni declares that the article meets the requirements of:

- Article 3, 11(5), 15 and 17 of Regulation (EC) No 1935/2004 (Framework regulation)

- EU Regulation 2023/2006/EC (GMP)

- EU Regulation 10/2011/EC with amendments (Plastic regulation)

Overall migration (1)

According to the above-mentioned regulation EU Regulation 10/2011, the overall migration does not exceed 10 mg/dm² or 60 mg/kg.

Specific migration (2)

This article contains monomers or additives subject to restrictions under the plastic regulation 10/2011 and its amendments. A specific migration test proves that these are within the limits. If more information regarding the monomers or additives are needed please contact Duni AB.

Area of use

Based on the migration tests and Declaration of Compliance, the article can be used safely with all types of foodstuffs except 100% fat or foods preserved in oil under the following application:

- Any long period storage at room temperature or below

- Including heating up to 70°c for up to 2 hours, or heating up to 100°C for up to 15 minutes

The film is not suitable to be used in a microwave oven.

Test conditions

Migration tests on the material of the article performed by an independent institute showed that under the following test conditions overall migration (see 1.) and specific migration (see 2.) fall below the respective limits given by regulation 10/2011.

Overall migration

3% Acetic Acid		2 hours at 100°C

10% Ethanol		2 hours at 100°C

Vegetable oil		1 hour at 121°C

Specific migration of metals

3% Acetic Acid		2 hours at 100°C

Specific migration of phthalates

Isooctane			2 hours at 60°C

The ratio of food contact surface area to volume used is 6 dm²/kg

No substances of dual use are present in the product.

According to the document in our possession, Primary Aromatic Amines are below 10 ppb.

Please be advised that Duni Group does not add anything into the product.

This document of compliance is based on:

-	Documentation from suppliers

-	Global migration test

-	Specific migration

This document was issued electronically and is therefore valid without signature.