DECLARATION OF COMPLIANCE

| Description | Material | Article Number |
| --- | --- | --- |
| TRAYS, FORMS, LIDS | PP | 161179 |

Duni declares that the article meets the requirements of:

- Article 3, 11(5), 15 and 17 of Regulation (EC) No 1935/2004 (Framework regulation)

- EU Regulation 2023/2006/EC (GMP)

- EU Regulation 10/2011/EC with amendments (Plastic regulation)

Overall migration (1)

According to the above-mentioned regulation EU Regulation 10/2011, the overall migration does not exceed 10 mg/dm² or 60 mg/kg.

Specific migration (2)

To our knowledge the product does not contain any monomers or additives with restrictions to regulation 10/2011/EC.

Area of use

Based on the migration tests and Declaration of Compliance in Duni’s possession, the articles can be used safely with all types of foodstuffs, serving cold and hot. The articles can be used for storage > 24 hours, hotfill and temperatures up to 70 °C for 2 hours.

The articles can be used in the microwave.

Test conditions

Migration tests on the material of the article performed by an independent institute showed that under the following test conditions overall migration (see 1.) and specific migration (see 2.) fall below the respective limits given by regulation 10/2011.

Overall migration	3% Acetic acid	10 days at 40°C and 2 hours at 100°C

10 % Ethanol	10 days at 40°C and 2 hours at 100°C

20 % Ethanol	10 days at 40°C and 2 hours at 100°C

50 % Ethanol	10 days at 40°C and 2 hours at 100°C

OM5*

OM5*

95 % Ethanol	10 days at 40°C and 3,5 hours at 60 °C

Isooctane		2 days at 20°C and 1,5 hours at 60°C

*corresponding standard testing conditions in 10/2011

The ratio of food contact surface area to volume used is 6 dm²/kg

No substances of dual use are present in the product.

Please be advised that Duni Group does not add anything into the product.

This document of compliance is based on:

-	Documentation from suppliers

-	Global migration test

-	Specific migration

This document was issued electronically and is therefore valid without signature.

Definition from COMMISSION REGULATION (EU) 2016/1416: “hot-fill” means the filling of any article with a food with a temperature not exceeding 100 °C at the moment of filling, after which the food cools down to 50 °C or below within 60 minutes, or to 30 °C or below within 150 minutes.